A little update on the 32u4

I have played a bit with various bootloaders. E.g. I have flashed the ATMEL DFU bootloader. After some hassle I could install the FLIP tool. That communicates with the 32u4 using libusb device driver. I can use this to flash the Thermoino code, BUT it will not expose a serial port as expected and is still in libusb mode, i.e. totally useless. I have now simply flashed the Caterina-A-Star.hex bootloader from Pololu. This now works. Funny observation: when first connected after flashing it exposed COM15. But after flashing the Thermoino code it was back to COM12. This then means I cannot flash another hex file…only works when pressing the reset button twice.

I will now try to burn the original Arduino bootloader.

It burns this file:

C:\Users\buechel\AppData\Local\Arduino15\packages\arduino\hardware\avr\1.8.6/bootloaders/caterina/Caterina-Leonardo.hex

It also does this: -Ulock:w:0x2F:m 
Now it has COM12: Then opening this, the Leonardo exposes COM13 for upload. As usual. Let’s see whether we can now flash again. Yes, works like a charm… So something seems to be broken with the A-Star Pololu bootloader.

Wow that’s weird. The problem seems to be the timing and maybe naming. I have now created a Powershell script that opens the normal port with 1200baud, then closes it and then waits for a new port to appear, which is then passed to AVRDUDE for upload. The script deliberately searches for names of the devices (as defined in CDC):

$deviceName     = "Pololu A-Star 32u4"

$bootloaderName = "Arduino Leonardo bootloader"

and makes a little pause between closing and opening:

# --- Wait for bootloader ---

Write-Host "Waiting for bootloader..."

$bootPort = $null

$timeout = [DateTime]::Now.AddSeconds(10)

while (-not $bootPort -and [DateTime]::Now -lt $timeout) {

Start-Sleep -Milliseconds 200

$bootPort = Get-ComPortByCaption $bootloaderName

}

This works very nicely. What is important is that the build environment is set to the bootloader. That’s because the startup sequence relies on the 32u4 to be approachable via CDC (serial protocol). The CDC code for original Leonardo and the A-star implementation is different (the VID:HID differ).

Let’s keep the Leonardo bootloader for now!